**********************************************************************

MIEKE - A Package of Programs for Neutron Spectrum

Adjustment based on "Few Channel" Measurements

**********************************************************************

M. Matzke

Physikalisch-Technische Bundesanstalt

D-3300 Braunschweig

Version March 1991, new comments added 1994

This is the manual for the program package MIEKE. The package contains a

lot of utility programs and some unfolding (adjustment) codes together with a

data base. It may be used either for BONNER sphere measurements or for

reactor dosimetry purposes.

The user should first follow the examples given, in order to learn how the

package is organized. After that he may use own measurements of reaction

rates together with the adjustment codes.

The package was developed as a contribution to the REFERENCE DATA

FILE FOR NEUTRON SPECTRUM ADJUSTMENT /1/. It contains data and

modified versions of programs of other authors, a preliminary version was

used during the REAL exercises /2,3,4/:

It contains also:

Modified versions of programs by H. J. Nolthenius and H. Ch. Rieffe, ECN

Petten /5,6,18/: XSC32F, UNC33, XSC33F, FITOCO, XSCOPY.

Data files:

IRDF-85-file by D. E. Cullen and P. K. McLaughlin /7/

(not longer in the present version)

IRDF-90-file by N. P. Kocherov and P. K. McLaughlin /8/

modified by E. J. Szondi /17/

RESPDAT (Response functions for BONNER spheres prepared by

A. V. Alevra /9/).

Sample input data of the REAL88 exercise /4/.

******************************************************************

The package can be used

for reactor dosimetry, with the set-up file RDSETUP.BAT

and for BONNER-sphere unfolding  with the set-up file BSSETUP.BAT

Attention: use the setup files to copy the data to your computer !

*************************************************************************

CONTENTS:

1.      INTRODUCTION TO LEAST-SQUARES ADJUSTMENT

2.      PROGRAM AND DATA SOURCES, SHORT DESCRIPTION

3.      GETTING STARTED

4.      EXAMPLE OF REACTOR DOSIMETRY: THE PS1 SPECTRUM

5.      OTHER EXAMPLES IN REACTOR DOSIMETRY

6.      EXAMPLE OF BONNER SPHERES

7.      FORMATS OF SPECTRUM FILES, REACTION RATE FILES AND

LIBRARIES

8.      MATHEMATICAL APPROACHES FOR MSANDB, MSITER AND MIEKE

9.      COMMENTS ON THE PROGRAMS

10.     REFERENCES

******************************************************************

*************************************************************************

1. INTRODUCTION TO LEAST-SQUARES ADJUSTMENT.

*************************************************************************

This program package deals with neutron spectrum adjustment based on

multiple foil reaction rate or BONNER spheres measurements. This is a so-

called "few channel unfolding" problem.

A large number M of neutron group fluence values

PHI(m) (m=1,...,M) has to be determined from the counted reaction events

A(i) (i=1,...,K) of a small number of K detectors. PHI(m) is the integral of the

spectral fluence integrated over an energy interval characterized by m.

Generally, the problem of determining the neutron fluence at a point of interest

is equivalent to the problem of combining all available information on physical

quantities (which are relevant to the point of interest) in order to get

"improved" results.

Such quantities could be:

A pre-information fluence vector F0 (with M elements) obtained e.g. by a

transport code calculation. It is assumed that the covariance matrix KF0 of F0

is known. Sometimes F0 can only be calculated relatively, i. e. the absolute

fluence is

PHI = C * F0 with an unknown scaling factor C.

A cross section matrix S0 consisting of K reaction cross sections with

elements S0(i,m) (i=1,...,K; m=1,...,M) with the covariance matrix KS0. S0 and

KS0 are taken from the literature. S0 is sometimes called a response matrix.

A vector A0 of measured reaction events (or reaction rates) for the K reactions

with the covariance matrix KA0.

Only within the STAY'SL code /10/and its modified version MSITER /11/ is a

consistent "combination" of all relevant quantities performed. The algorithm is

described in the following:

By a consistent "combination" of relevant quantities it is intended to

determine new (improved) values F, S, A (and C). Between these values a model

equation exists:

A = C * S * F                                                               (1)

Representing the pre-information values S0 and F0 by a so-called parameter

vector P0 /10,12/ with M + K*M elements, the determination of C, S, F consists

in minimizing a Chi-squared expression:

CHI**2  =  (A0 - A)*KA0(-1)*(A0 - A) + (P0 - P)*KP0(-1)*(P0 - P) +

2*(A - C * H(P)) * L                                                        (2)

In Eq.(2)  KA0(-1) and KP0(-1) are matrix inverses,

A=C*H(P) is the model equation (1) and L is the Lagrangian multiplier/12/

vector. The factor of 2 is introduced for convenience. Left-handed vectors in

Eq. (2) are considered as transposed.

Minimization of CHI**2 with respect to A, P, C together with the constraint

A=C*H(P) constitute K+M*K+K+K+1 equations for the unknown A, P, L, C.

The model equations A=C*H(P) are non-linear leading to non-linear normal

equations during the minimization process. Generally the solution can be

obtained by iteration. For thist, it is assumed that C*H(P) can be expanded into

a Taylor series in the neighbourhood of fixed (first approximation) values C1

and P1:

C*H(P) = C1*H1 + H1*(C-C1) + C1*HP*(P-P1) +....                             (3)

with H1=H(P1)=S1*F1 (K elements). The derivative HP = dH/dP is taken at

P=P1.

Truncating the taylor serie, the minimization leads to the equations:

A = A0 - KA0*L

P = P0 + C1*KP0*HP*L                                                        (4)

H1*L = 0

Taking into account the model equations (3) the following solution is obtained

for one iteration step:

L = (W - W*T*W) * X

C = C1 + H1*W*X / R                                                         (5)

where W is the matrix inverse of KA0+C1*C1*HP*KP0*HP,

X = A0 - C1*H1 - C1*HP*(P0-P1),

R = H1*W*H1 (pure number),

T = H1 H1/R is a dyadic matrix.

In the iteration procedure the pre-information values P0 are taken as first

approximates together with an estimated value C1. The iteration is continued

by using the solutions of Eq. (4) and (5) as new approximates P1 and C1.

In the linear least-squares adjustment only the first step of the iteration is

carried out /10,12/, which is equivalent to the minimization of CHI**2 with

model equations:

A = C*H0 + C1 * HP*(P-P0)                                                   (6)

with H0 = H(P0) = S0*F0 (K elements)

This linear least-squares method leads to solution equations similar to Eqs. (4)

and (5), where P1 is now replaced by P0 and

X = A0 - C1*H0.

In the STAY'SL code the scaling factor C is assumed to be known, therefore

no minimization is performed with respect to C. In that case the solution is

similar to Eqs. (4) and (5), but the terms containing the matrix T are missing.

The program MSITER of this package allows either a linear or a non-linear

solution of the minimization equations.

------------------------------------------------------------------

***************************************************************

2. PROGRAM AND DATA SOURCES, SHORT DESCRIPTION

***************************************************************

The MIEKE package contains three unfolding (adjustment) codes written by

the author:

MSANDB          The algorithm is taken from the original SAND-II code by

Mc Elroy /13/, however, uncertainties on the reaction rates

can be taken into account.

MSITER          This is a modification of the original STAY'SL algorithm

of F. G. Perey /10,11/.

In addition to the STAY'SL code the normalization factor

of the spectrum can be calculated by MSITER.

MIEKEB          Unfolding code for cases where there is a lack of pre-

information on the spectrum. MONTE CARLO code written

by the author/14/.

A detailed description of the algorithms is given by the author in reference /16/.

The codes can be used for unfolding or adjusting measured counting rates

(reaction rates, activities, reaction events) in reactor dosimetry as well as for

the unfolding of counting rates of BONNER spheres.

Various utility programs must to be used beforehand to prepare the input files

for the adjustment codes.

2.1     UTILITY PROGRAMS FOR CROSS SECTIONS AND RESPONSE

FUNCTIONS

--------------------------------------------------------------

2.1.1 Bonner Spheres

--------------------

The original response functions are contained in the RESPDAT file. They have

been evaluated by A. Alevra (PTB) /9/ and should be considered as

preliminary. (Private Comm., to be published). The response functions in

RESPDAT can be transformed to a library in SAND-II format (DOSCROSS

format /15/) by the program MRESBI.

2.1.2 IRDF-File

---------------

The IRDF-90 file (N. P. Kocherov and P. K. McLaughlin /8/) has been revised by E.

J. Szondi /17/. The present version (IRDF-90/NMF-G) is included in this package.

The name of this file is IRDF90NG.dat. It includes cross section sets and

covariance data on 53 reactions of 37 detector materials and 9 cross section sets

without covariance information (cover materials, dpa values).

The ENDF formatted data can be transformed to a SAND-II 640 group library

of DOSCROSS /15/ type by the program MCROBI.

Several statements from the utility programs of the REAL exercises /4/ are

copied to MCROBI.

For the handling of the covariance information on the cross sections the

programs

XSC32F, UNC33, XSCOPY, XSC33F /6, 18/ of the REAL88 exercise /4/ have been

transformed to FORTRAN 77 and included in new programs XSC32M90, UNC33M,

UMSBIB. Program XSC33F is no longer needed, its output is included in the

new UNC33M.

XSC32F, renamed XSC32M90, is only needed for the covariance data of

NP237F, MAT=6337 of the IRDF-85 file /7/, it is not needed for the data of

IRDF-90.

With the program UNC33M, cross section covariance matrices for a given

group structure are evaluated using MF=33 values in ENDF-B/VI format of the

IRDF-90 file.

It has been found that for many reactions, the variances obtained from the

IRDF-90 file are obviously too small, possibly a result of the fact that UNC33M

cannot take into account cross correlations.

2.2 Other utility programs

--------------------------

MSPECT          creates a fluence file in a certain group structure

for:

WATT spectrum

or NBS-Uranium fission spectrum

or NBS-Californium fission spectrum

or user defined spectrum.

UMSBIB          transforms a formatted SAND-II library into a binary

(old program XSCOPY of REAL88 /4,18/). This program is only

needed if new formatted cross section parts have to

be copied to the formatted part of the library after

a MCROBI or a MRESBI run.

UMSDAT          transforms a data file of the REAL88 /4/ exercise or

the REFERENCE DATA FILE /1/ to the user format needed

in this package.

MFITIB          transforms fine-structure libraries and fluences to

coarse structures and performs shielding calculations

for cross sections and response functions.

A lot of statements from the program FITOCO /5/ are

included in that program

PLOT            plots screen graphics (see below). The program plots

curves on screen and HP-plotter or LASER JET printer

after the run of most of the programs.

PLOF            plots spectrum files and cross section data from IRDF-90

or from a formatted or unformatted DOSCROSS library.

For PLOF and PLOT:

A plotting file can be created which can be copied to a

COM output for HP-plotter or to a Laser-Jet output

(for Laser Jet: PCL commands are added to the HPGL file)

The files can be used as input for MICROSOFT WORD.

------------------------------------------------------------------------

**************************************

3. GETTING STARTED:

**************************************

Please follow the instructions given :

----    create a new directory on your harddisk

----    make this directory to your current directory

----    Insert diskette no.1 into drive A

For reactor dosimetry work:

+++++++++++++++++++++++++++

----    Copy the file RDSETUP.BAT from drive A to the new directory

----    Type RDSETUP %1 %2 and follow the instructions given

%1 means drive of source diskette e. g. A:

%2 means drive (or) directory of destiantion e. g. D:

For BONNER sphere work:

+++++++++++++++++++++++

----    Copy the file BSSETUP.BAT from drive A to the new directory

----    Type BSSETUP %1 %2 and follow the instructions given

%1 means drive of source diskette e. g. A:

%2 means drive (or) directory of destiantion e. g. D:

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

You now have the programs with the data needed for the example and the

plotting routines in your directory.

Before starting you have to copy the .EXE files contained in LHA- compressed

form on DISKETTE No. 2 from the directory \Programs to a directory on your

harddisk.

Decompress: by typing PROGRAMS.EXE

If you do not intend to use the .EXE files, you may compile by yourself.

Before starting you have to change the compiling and linking file

COMPLINK.BAT

Change the drives and environment settings for compiling and linking to your

own options.

The programs should run with a MICROSOFT FORTRAN version larger than

3.3. If you use an older version you might have difficulties with the file-OPEN

statement.

The programs were tested with MICROSOFT FORTRAN 5.0, and the

MICROSOFT POWER STATION COMPILER where the default value in the

OPEN statement of the parameter STATUS is: 'UNKNOWN'.

In some of the programs the STATUS parameter 'SCRATCH' is used, which is

not understood by the version 3.3

The programs have been compiled and linked with the Gt attribute (Look at the

file COMPLINK.BAT). For older compilers you may include the $LARGE

Metacommand in the first line of your programs if you have storage problems.

In some of the programs a subroutine PLBILD is used to write the results to a

file PLOTOB. After a program run this file can be converted to a screen

graphics output by typing PLOT. This program works with HERCULES, EGA

and VGA videomodes.

Note added 1994: Due to problems with binary files produced with non

MICROSOFT compilers, the non-formatted file PLOTOB is replaced here in the

present version by an ASCI file PLOTASC, which is now used by the

programs PLOT and PLOF.

For HERCULES graphic you have first to type MSHERC.

The screen output can be modified by the CH (change) parameter at the run

stage.

You are prompted to a maximum of 16 parameters like text for x/y axis, tick-

length, factorization of the curves and plotting symbols. You may also change

the output file name (default: COM1) for HP to an arbitrary file name or to

COM2. The HP plotting should work with the following settings in

AUTOEXEC.BAT:

MODE COM1:96,e,7,1,p

(used HP-plotter: HP7475A)

Microsoft does not allow the distribution of graphic fonts, so you must first

transfer the text fonts available in your computer to graphic fonts.

This is performed by the ERZFON program, which has been copied to the

current directory. For the HERCULES mode type MSHERC before typing

ERZFON. After the run of ERZFON you have the font file named MATZ.FON

available in the current directory.

For plotting ENDF (IRDF-90) cross sections or cross sections in DOSCROSS

format from a formatted or unformatted file or for plotting spectrum files, you

may use the PLOF program which has been copied to your directory. You are

prompted to the file name and a number of parameters.

All the programs (with the exception of the plotting routines) use FILE 5 as

input which is assigned to CON.

Running the programs on other computers (e.g. the CDC), you must modify

the programs or assign to the file name CON the input file at the run stage.

The input file is in every case expected at unit No. 5.

If you have problems with compiling and linking, the EXE versions of all the

programs are included in the diskettes.

Due to problems with other compilers in this version of the programs not a

binary file PLOTOB but a ASCI file PLOTASC is produced, which is used by

the program PLOT.

If you have problems with binary files produced by other compilers (e.g. the

binary response matrix file) you may plot only the formatted response matrix

file.

------------------------------------------------------------------

CONTENTS OF THE DISKETTES:

------------------------------------------------------------------

-----------------------Diskette No. 1:

Directory PATH listing

\PLOT

\FORTRAN        (source codes)

\BONNIN         (input data for the BONNER sphere example)

\BONNOUT        (output data for the BONNER sphere example)

\IRDF1

\REFDATA

\INPUT

\INPUTANO

\INPUTPS1

\INPUTPS2

\INPUTRTN

\INPUTU35

\INPUTCFR

ROOT DIRECTORY:

README.DOC      This document

READ1.DOC       Short introduction

RDSETUP.BAT     Set-up file for reactor dosimetry work

BSSETUP.BAT     Set-up file for BONNER spheres work

COMPLINK.BAT    Compiling and linking file, look at it and

change it for your compiler and path options !!!

BATCHRD.BAT     Batch file for an example of the REFERENCE

DATA FILE.

BATCHBS.BAT     Batch file for a BONNER sphere example.

Directory of  A:\PLOT:

ERZFON.EXE      Program to create your graphic fonts

PLOT.EXE        Program to plot the results of your runs

immediately after a run

PLOF.EXE        Program to plot IRDF(ENDF) data or a spectrum file

MSHERC.COM      Microsoft program to be used for HERCULES card

before plotting (initializing HERCULES)

Directory of  A:\FORTRAN:

ENDF.FOR        Produces a listing of MAT and MT numbers contained in a ENDF file

(like IRDF90Ng.dat).

MSPECT.FOR      Creates a spectrum file in standard format.

MRESBI.FOR      Creates the response ("cross section")

library for BONNER spheres

UMSBIB.FOR      Transforms a formatted library to an unformatted.

This program is not needed for the examples given.

MFITIB.FOR      Creates a library (formatted and unformatted)

in coarse energy structure and performs self-shielding

and shielding calculations.

MSANDB.FOR      Adjustment program

MSITER.FOR      Adjustment program

MIEKEB.FOR      Adjustment program

MCROBI.FOR      Creates a fine group library from IRDF

UMSDAT.FOR      Transforms data given in the data files  *.DAN

to data files of MIEKE-format

XSC32M90.FOR    Calculates the MF=32 part of the cross

section covariance matrix for NP237F

UNC33M.FOR      calculates cross section covariance matrices coarse

energy structure from IRDF data (ENDF-B/VI format).

Directory of  A:\BONNIN:        (BONNER spheres input files)

ACT             Activity (reaction rate) file, change the numbers,

if you have your own measurements.

RESPDAT         Response functions by A.V. Alevra

Input files for the programs:

MSPECT.INP

MRESBI.INP

MSANDB.INP

MSITER.INP

MIEKEB.INP

MIEKEB.IN2

Directory of  A:\BONNOUT:       (BONNER sphere output files)

MSPECT.OUT

RESP48.FMT

RESP48.BIN

MRESBI.48

MRESBI.PR1

MSANDB.PR1

FLUX48.SAN

MSITER.PR1

FLUX48.MS

FLUXC48.MS

MIEKEB.PR1

FLUX48.MI1

FLUXC48.MI1

MIEKEB.PR2

FLUX48.MI2

Directory of  A:\IRDF1: (IRDF-85 resonance data and IRDFDAM of IRDF-90)

IRDFDAM         Damage cross setion sets of the IRDF-90 file /8/.

NEPT.COV        Covariance part for MF=32 for NP237F MAT=6337 from REAL-88 /3/

XSC1            Example output for the PS1 example from program XSC32M90.FOR

needed by UNC33M /3/

These files are not needed for the present version of the data.

Directory of  A:\REFDATA:       (Reference data files)

PS1.DAN

RTN.DAN

Note: The other data sets of the Reference Data File

are included in disk No. 3

Directory of  A:\INPUT:         (input files for MCROBI)

MCROBI.INP

MCROBI.DAT      assigning reaction names to MAT-MT numbers

MSPECT.INP      Input file for the program MSPECT

Directory of  A:\INPUTANO:      (Input files for the ANO example)

ACT.ANO

UMSDATIN.ANO

UNC33MIN.ANO

MFITIBIN.ANO

MSANDBIN.ANO            remark: The activities for the ANO example

are multiplied by 1.0e8 to avoid overflow

MSITERIN.ANO.

MIEKEBIN.ANO

MIEKEBI2.ANO

Directory of  A:\INPUTPS1:      (Input files for the PS1 example)

ACT.PS1

UMSDATIN.PS1

UNC33MIN.PS1

MFITIBIN.PS1

MSANDBIN.PS1

MSITERIN.PS1

MIEKEBIN.PS1

MIEKEBI2.PS1

Directory of  A:\INPUTPS2:      (Input files for the PS2 example)

ACT.PS2

UMSDATIN.PS2

UNC33MIN.PS2

MFITIBIN.PS2

MSANDBIN.PS2

MSITERIN.PS2

MIEKEBIN.PS2

MIEKEBI2.PS2

Directory of  A:\INPUTRTN:      (Input files for the RTN example)

ACT.RTN

UMSDATIN.RTN

UNC33MIN.RTN

MFITIBIN.RTN

MSANDBIN.RTN

MSITERIN.RTN

MIEKEBIN.RTN

MIEKEBI2.RTN

Directory of  A:\INPUTU35:      (Input files for the U35 example)

ACT.U35

UMSDATIN.U35

UNC33MIN.U35

MFITIBIN.U35

MSANDBIN.U35

MSITERIN.U35

MIEKEBIN.U35

MIEKEBI2.U35

Directory of  A:\INPUTCFR:      (Input files for the CFR example)

ACT.CFR

UMSDATIN.CFR

UNC33MIN.CFR

MFITIBIN.CFR

MSANDBIN.CFR

MSITERIN.CFR

MIEKEBIN.CFR

MIEKEBI2.CFR

-------Diskette No. 2:

Directory PATH listing

\IRDFLIB

\PS1OUT

\PROGRAMS

Directory of  A:\IRDFLIB:       (output files from MCROBI)

IRDF.FMT                        Formatted 640 group library

IRDF.BIN                        Unformatted 640 group library

Directory of  A:\PS1OUT:        (Output files for the PS1 example)

UMSDATPR.PS1

UNC33MPR.PS1

COV.PS1

CROSSBI.PS1

MFITIBPR.PS1

CROSS1BI.PS1

MSANDBPR.PS1

MSITERPR.PS1

ACT2.PS1

COVACT.PS1

F1.PS1

CF.PS1

W1.PS1

ACT.PS1

CROSS.PS1

CROSS1.PS1

FF1.PS1

FSAND.PS1

FSITER.PS1

CFSITER.PS1

MIEKEBPR.PS1

FMIEKE1.PS1

CFMIEKE1.PS1

MIEKEBP2.PS1

FMIEKE2.PS1

CFMIEKE2.PS1

Directory of  A:\PROGRAMS       (EXE files in compressed form)

PROGRAMS.EXE            (type PROGRAMS.EXE to decompress)

------Diskette No. 3 :

IRDF90NG.DAT    IRDF-90/NMF-G file by E. J. Szondi /17/.

\REFDATA

Directory of  A:\REFDATA:       (Reference data files)

ANO.DAN

PS1.DAN

PS2.DAN

U35.DNC (note: this file is renamed U35.DAN when copied to the current directory)

CFR.DAN (note: the weighting spectrum is in units fluence per lethargy)

********************************************************************

4.  EXAMPLE OF REACTOR DOSIMETRY: THE PS1 SPECTRUM

********************************************************************

There is a batch file named BATCHRD.BAT to run all the programs for all the

examples of the REFERENCE DATA FILE:

BATCHRD.BAT %1 %2 %3.:

Admitted values for the parameter %1 are:

ANO

PS1

PS2

RTN

U35

CFR

Parameters %2 and %3 are optional.

If %2=p or %2=P or %2 =PLOT or %2=plot, the program PLOT is started after

each program run to display the results on screen or to use HP plotter.

The parameter %3 may be used for the path where the EXE files of the

programs are found.

Before using the batch file all the EXE files should be made available.

Example: BATCHRD PS1 PLOT c:\fortran\

It is worthwhile to run the programs step by step. An example of the PS1

evaluation is described in the following.

Please follow the instructions in the order given.

In the following example, the extension .PS1 is used for all special input

and output files.

************************** UMSDAT ******************************

Transform the data of the PS1 example (PS1.DAN) to the formats of the

MIEKE

package and create the data files needed in the following adjustment:

type:   COMPLINK UMSDAT         to compile and link the program

type:   UMSDAT < UMSDATIN.PS1   which means that you use

UMSDATIN.PS1 as the input

file instead of CON input.

All the names of the output files

are listed In the input file. Look at

it to learn more about the

the organization.

After the program has finished you have the following files available:

UMSDATPR.PS1            The printer output,

ACT2.PS1                The file for reaction

events or reaction rates,

COVACT.PS1              Covariance matrix of reaction events,

F1.PS1                  Fluence file of PS1.DAN,

CF.PS1                  Covariance matrix of the fluence,

W1.PS1                  Weighting spectrum.

type:   PLOT        to look at the screen graphics. (before this, type MSHERC

if you have HERCULES adapter)

You may leave the screen graphics by typing: OK

or by typing END

look at the printer output file UMSDATPR.PS1

****************************** MSPECT ****************************

This program creates a fluence file in the special SAND-II energy group

structure. The program is in principle not needed for the following codes but it

is sometimes wanted to use a special new spectrum as a pre-information in

adjustment codes. Such a spectrum can be created by MSPECT.

In the present example the MSPECT output file is used by the MCROBI program.

For MSPECT the following options are possible:

MFLUX=0       special user defined spectrum, look at the program.

MFLUX=1       Watt spectrum

MFLUX=2       NBS Uranium fission spectrum

MFLUX=3       NBS Californium fission spectrum.c

MFLUX=4       NBS URANIUM FISSION + 1/E PART, MATCHED AT 0.8 MEV

MFLUX=5       the same as MFLUX=4 but thermal spectrum (MAXWELLIAN)

used  below 0.1ev

For more details look at the program.

type:     COMPLINK MSPECT     for compiling and linking

type:     MSPECT < MSPECT.INP to run the program with the input

file MSPECT.INP

You now have the fluence file MSPECT.OUT available (WATT spectrum)

This spectrum can be used by the following program MCROBI.

type:     PLOT                to look at the spectrum file.

************************** MCROBI ********************************

This is a general program which has nothing to do with the PS1 problem. It

converts the IRDF-90 file to a library of SAND-II type. The format of this

library is the same as in the CS640 file of the REAL88 exercise /4/.

The file IRDF90NG.DAT which is a modified IRDF-90 file /17/, has been copied to

your directory.

input files needed :

MCROBI.INP          Look at this file,

IRDF90NG.DAT        modified IRDF-90 file

MCROBI.DAT          A data file, where the connection between

MAT-MT-numbers and the names

of the reactions is listed. The names are the same as used

in the activity file.

MSPECT.OUT          A 640 group fluence file is required the program calculates

the integral cross sections for that spectrum and writes the

results to the printer file. In the present example

MSPECT.OUT is the output of a MSPECT run. Instead of this,

you may use e.g. W1.PS, i.e. the weighting spectrum from the

UMSDAT run. In that case you should modify the input file

MCROBI.INP

type:     COMPLINK MCROBI     for compiling and linking

type:     MCROBI < MCROBI.INP for running the program with the

input file MCROBI.INP.

Look at the input file

and the data file MCROBI.DAT

You have now the following files available:

IRDF.FMT            Formatted IRDF library in SAND-II format

MCROBI.PR1          Printer output file

IRDF.BIN            Unformatted IRDF library.

type:     PLOT                to check what you have done

In IRDF.FMT and IRDF.BIN several additional 'cross sections' termed dosimetry

reactions or 'Pseudo cross sections'are created, which may be used as input

for the adjustment codes, namely:

SHORT NAME   16-CHARACTER NAME

PHI-THER   PHI-THERMAL*****     ENERGIES E < 0.563E-6 MEV

PHI-INT*   PHI-INTERMEDIATE     0.563E-6 < E < 1.05 MEV

PHI-0.1*   PHI < 0.1 MEV **     E > 0.1 MEV

PHI-1.0*   PHI < 1.0 MEV **     E > 1.0 MEV

PHI-TOT*   PHI-TOTAL ******     TOTAL FLUX

< E > **   MEAN ENERGY <E>*

TO 0.5EV   THERM TO 0.5 EV      E < 0.5 EV

5E-7 E-3   0.5 EV TO 1  KEV     5.0E-7 MEV < E < 1.0E-3 MEV

E-3  E-1   1  KEV TO 100KEV     1.0E-3 MEV < E < 0.1 MEV

0.1  1.0   100KEV TO 1.0MEV     0.1 MEV < E < 1.0 MEV

1.0  5.0   1.0MEV TO 5.0MEV     1.0 MEV < E < 5.0 MEV

FROM 5.0   5.0MEV TO 20 MEV     5.0 MEV < E

1/V*       1/V RESPONSE FKT

Look at the printer output file MCROBI.PR1

************************* XSC32M90********************************

General program for the creation of the Neptunium covariance file for MAT=6337

of IRDF-85.

Not directly attached to the PS1 example considered. It calculates MF=32

contributions to the covariance matrix of neptunium. It is a modified

utility program of the REAL exercises /4/.

It makes a direct access library of the cross section covariance matrix

of type MF=32.

You need this program only for cases in which the NP237F reaction has

been measured.

It is not needed for the data if IRDF-90.

input files:        TAPE35 = nept.cov

(NP237F COVARIANCE MATRIX IN COMPRESSED FORM)

type:     COMPLINK XSC32M90   for compiling and linking

type:     XSC32M90            to run the program. You need no other input file;

automatic open statements with the name

given above (NEPT.COV).

You now have time for an extended lunch, depending on the speed of your

computer. One hour is needed for a COMPAQ 386/20. If your computer was cheaper

and is not so fast, you have saved more money for a more extended lunch.

output:

TAPE6  = LINE PRINTER OUTPUT

TAPE16 = NEW LIBRARY part for NP239F named :   XSC1

note:     the file XSC1 is required as input file in UNC33M if

NP237F, MAT=6337 is part of the measured reactions

no plotting is possible

*************************  UNC33M ********************************

Modified utility program /6,18/ to create covariance matrices. It converts

ENDF covariance data contained in IRDF-90 to the special format of the

MIEKE package.

A UNC33M run is needed for every activity file and for

every new coarse energy structure.

input files:

UNC33MIN.PS1        Input file containing all

information about the files used.

XSC1                Output of XSC32F (optional).

There is a switch variable requested

from input for cases where XSC1 is

not needed. See UNC33MIN.PS1.

ACT.PS1             This is the output file of UMSDAT.

In the example considered here the UMSDAT

output file was termed ACT2.PS1. The file ACT.PS1

is nearly the same but already contains

the shielding information (see MFITIB).

This file was copied from the diskette to

your directory.

F1.PS1              Output file of UMSDAT containing the coarse groups,

IRDF.BIN            Output file of MCROBI containing the binary

IRDF library.

W1.PS1              Output file of UMSDAT containing the weighting

spectrum. There is a switch variable requested

from input for the option of 1/E weighting.

See UNC33MIN.PS1 and the program description.

IRDF90NG.DAT        Modified IRDF file.

type:     COMPLINK UNC33M     for compiling and linking

type:     UNC33M < UNC33MIN.PS1        to run the program with the

input file UNC33MIN.PS1.

You have now time for an extended dinner, 1 hour is needed for COMPAQ-386/20.

On your return you will find (hopefully) the following files:

UNC33MPR.PS1        Printer output file.

COV.PS1             Cross section covariance file

CROSS.PS1           Formatted cross section file in

coarse structure for PS1; the energy

structure is defined by F1.PS1.

CROSSBI.PS1         The same file as CROSS.PS1 but unformatted.

no plotting possible.

Remark: If you do not have to perform shielding operations, you may use the

file CROSSBI.PS1 as cross section input file for the adjustment codes.

Shielding and self-shielding is performed by the program MFITIB (see below).

********************* MFITIB *************************************

Modified program FITOCO /5/ of the REAL exercises /4/.

The program makes a formatted output of coarse group cross sections similar

to the program UNC33M using the 640 group binary cross section file IRDF.BIN

In addition to FITOCO and UNC33M, several options for the program are

possible:

MMODE=1 :  The new coarse group structure is read from a file.

=2 :  The new group structure is evaluated from a fine

structure file, so that the group fluence rate values

FLUX*DE (DE: ENERGY INTERVAL) in all groups are

identical, the number of the new groups is requested from

the input file.

MMODE=3 :  The new group structure is evaluated in the same way as

MMODE=2, but now the values FLUX*DE*H(E)

(H(E): a 'weighting cross section', the name of which must be part

of the binary library) are equal for all groups.

MMODE=4 :  The coarse group structure is calculated using input

values requested from the input file.

The program also performs a shielding calculation if in the activity file

the shielding switch variable following the number of reactions is set

equal to 1 or 2 (see description of the program). Beam incidence (iso=1)

and isotropic incidence (iso=2) of neutrons can be taken into account.

In the case of shielding the cover name from the binary library must follow

the MAT-MT numbers in the activity file. The cover name is followed by the

'thickness' of the cover in units per barn. Have another look at the

activity file to learn more about its structure.

input files:

MFITIBIN.PS1        Input file containing all information.

W1.PS1              Weighting spectrum, output from UMSDAT

IRDF.BIN            Binary fine cross section file,

in 640 groups, ouput from MRCOBI

ACT.PS1             Activity file

F1.PS1              Coarse energy fluence file, output from UMSDAT.

type:     COMPLINK MFITIB     for compiling and linking,

type:     MFITIB < MFITIBIN.PS1

to run the program with the input

file MFITIBIN.PS1

After the run you have the following files available:

MFITIBPR.PS1        Printer output file

CROSS1.PS1          (Shielded) Formatted cross section

file  in the coarse group structure.

CROSS1BI.PS1        The same file but unformatted.

If no shielding calculation is

performed, the values of the cross

sections should agree with those of

the cross section otput files

CROSS.PS1 and CROSSBI.PS1 from

UNC33M.

FF1.PS1             Coarse group fluence file,

transformed from the weighting

spectrum W1.PS1 (not used further)

type:     PLOT                To check the results.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Now you are able to perform the adjustment runs.

You have available:

Pre-information fluence file           :  F1.PS1 from UMSDAT

Pre-information fluence covariance file:  CF.PS1 from UMSDAT

Activity file (including MAT numbers)  :  ACT.PS1from UMSDAT (see comment on

this file above (at UNC33M))

Covariance file of activities          :  COVACT.PS1 from UMSDAT

Coarse (shielded) binary cross sections:  CROSS1BI.PS1 from MFITIB

Covariance file for the cross sections :  COV.PS1 from UNC33M

You may plot spectrum files (F1.PS1) or cross sections from the cross section

files (e.g. IRDF90NG.DAT, IRDF.BIN, IRDF.FMT, CROSS1BI.PS1) by typing:

PLOF

and following the instructions given.

**************************** MSANDB *****************************

Modified SAND-II program.

The uncertainties of the reaction rates are taken into account.

input files:

MSANDBIN.PS1        containing all the information

ACT.PS1             activity file

CROSS1BI.PS1        binary cross section file

type:     COMPLINK MSANB      for compiling and linking

type:     MSANDB < MSANDBIN.PS1

to run the program with the input

file MSANDBIN.PS1

You now have available:

MSANDBPR.PS1        Printer output

FSAND.PS1           output fluence file

type:     PLOT                to check the results

Note: You may also use the 640 group binary library for SAND-II unfolding

by changing the name (cross1bi.ps1) of the cross section library to IRDF.BIN

in the input file MSANDBIN.PS1 (or typing IRDF.BIN when using terminal input).

************************* MSITER *********************************

The algorithm of this program is the same as that realized in Francis Perey's

STAY'SL code /10/, but some additional options are included. The most

important change is the fitting of a normalization factor of the spectrum

in the framework of a non-linear least-squares procedure.

There is a switch variable NSTAY to tell the program which mode the user

prefers:

nstay=0 : non-linear iteration, normalization constant is

calculated in each iteration,

=1 : staysl run with normalization constant c=1,

=2 : staysl run, c is calculated once according to the algorithm

used in the STAY'SL program.

For more details look at the description of the program.

input files:

MSITERIN.PS1        Input file containing all

information.

In the PS1 case considered, NSTAY in

MSITERIN.PS1 is set to zero. i.e.

non-linear adjustment.

You may compare the results with those obtained

in the original STAY'SL code by setting NSTAY=2

in MSITERIN.PS1

ACT.PS1             Activity file

COVACT.PS1          Covariance matrix of activities

CROSS1BI.PS1        Unformatted coarse group cross

section file

COV.PS1             Covariance matrix of cross sections

F1.PS1              Input pre-information fluence file

CF.PS1              Covariance matrix of input fluence

type:     COMPLINK MSITER     for compiling and linking

type:     MSITER < MSITERIN.PS1

to run the program with the input

file MSITERIN.PS1

After the run you have the following files available:

MSITERPR.PS1        Printer output file,

FSITER.PS1          Fluence output file,

CFSITER.PS1         Binary output covariance file.

The covariance matrix of the output fluence can also be found in the printer

output file.

type:     PLOT                to check the results.

**************************** MIEKEB ******************************

Monte Carlo unfolding when no pre-information on the fluence is available

Warning: In few-channel unfolding you cannot pull rabbits out of a hat. The

output fluence of the MIEKEB must therefore be considered with some caution.

In most cases the relative uncertainty of the fluence is more than 100%.

However, the output integral values and their correlations provide a lot of

useful information.

The organization of the program is similar to MSITER.

You should perform 2 runs: The first one is only to check the temperature

parameter. The second (long-time) run takes many hours.

First run:

Input files:

MIEKEBIN.PS1        Input file containing all information

ACT.PS1

COVACT.PS1

CROSS1BI.PS1        see above for an explanation of these files.

COV.PS1

type:     COMPLINK MIEKEB     for compiling and linking.

type:     MIEKEB < MIEKEBIN.PS1

to run the program with the input

file MIEKEGIN.PS1

After the run you have the following files available:

MIEKEBPR.PS1        Printer output file,

FMIEKE1.PS1         Fluence output file,

CFMIEKE1.PS1        Binary output covariance file.

The covariance matrix of the output fluence can also be found in the printer

output file.

type:     PLOT                to check the results.

Second run:

type:     MIEKEB < MIEKEBI2.PS1

after the second run:

type:     PLOT

****************************************************************************

****************************************************

5. Other Examples in Reactor dosimetry:

****************************************************

If you are want to use the batch file BATCHRD.BAT mentioned in chapter 4

you type:

BATCHRD %1 %2 %3 ( %2, %3 optional) with the parameters stated in the

first lines of chapter 4.

e.g. : BATCHRD U35 PLOT C:\exe\

(running U35 data, plotting, EXE-files found in C:\exe\)

Otherwise you may continue as follows:

Example  U35 fission spectrum :

copy  U35.DNC  to your directory, rename it U35.DAN

You must now change the contents of the input files.

Note:

***************************************************************

All the input files mentioned in the following are available on

diskette No.1, there is no editing of the files necessary.

***************************************************************

copy file UMSDATIN.PS1        to UMSDATIN.U35

In the new file UMSDATIN.U35 change all the characters PS1 to U35.

The new file UMSDATIN.U35 now looks as follows:

umsdatpr.u35      comment:    printer output file

u35.dnc                       input data file

act2.u35                      output activity file

covact.u35                    output covariance of activities

f1.u35                        output fluence file

cf.u35                        output covariance of fluence

w1.u35                        output weighting spectrum

0                             switch (group fluence values )

640                           number of groups for weighting spectrum

1                             switch (fluence rate per energy)

type:     UMSDAT < UMSDATIN.U35

type:     PLOT

Copy UNC33IN.PS1   to UNC33IN.U35 and change all extensions PS1 to

U35.

Having copied all the input files  *IN.PS1   to *IN.U35, and having changed

*.PS1 to *.U35 in all the input files, you run the following programs:

UNC33M

MFITIB

MSANDB

MSITER

MIEKEB   with MIEKEBIN.U35

MIEKEB   with MIEKEBI2.u35

For calculations using your own activity measurements and pre-information

spectra, you create your own activity file and fluence file and fluence

covariance files.

Change all input files to the new names. If you have no pre-information

fluence change the relevant parameters in the input for MFITIB, MSITER

and UNC33M.

---------------------------------------------------------------------------

****************************************

6.  EXAMPLE OF BONNER SPHERES

****************************************

You may use the batch file BATCHBS.BAT %1 %2, where the optional

parameters %1 (for plotting) and %2 (path for EXE files) are explained in

chapter 4.

You get better information about the organization of the programs with the

following example:

The measured reaction rates are expected in file ACT. Look at it and

change it, if you have your own data.

The following names (8 characters) can be used to characterize a reaction:

DOSE-EQU          means:             DOSE-EQUIVALENT*

BARE****                             BARE COUNTER ***

2.0 IN**                             2.0 INCH *******

3.0 IN**                             3.0 INCH *******

5.0 IN**                             5.0 INCH *******

8.0 IN**                             8.0 INCH *******

10.0 IN*                             10.0 INCH ******

12.0 IN*                             12.0 INCH ******

18.0 IN*                             18.0 INCH ******

In addition, pseudo reaction rates can be calculated by the programs for the

following 'reactions':

PHI-THER             means:          PHI-THERMAL*****

PHI-INT*                             PHI-INTERMEDIATE

PHI-0.1*                             PHI < 0.1 MEV **

PHI-1.0*                             PHI < 1.0 MEV **

PHI-TOT*                             PHI-TOTAL ******

< E > **                             MEAN ENERGY <E>*

1/V*                                 1/V RESPONSE FKT

TO 0.5EV                             THERM TO 0.5 EV

5E-7 E-3                             0.5 EV TO 1  KEV

E-3  E-1                             1  KEV TO 100KEV

0.1  1.0                             100KEV TO 1.0MEV

1.0  5.0                             1.0MEV TO 5.0MEV

FROM 5.0                             5.0MEV TO 20 MEV

First you should create a spectrum file which is needed by the library

program MRESBI to calculate integral values of reaction rates. This file is

created by MSPECT:

****************************** MSPECT ****************************

This program creates a fluence file in the special SAND-II energy group

structure. The program is not needed for the unfolding codes, however, since

these codes and the library codes need a fluence file, it is worthwile to

first have a MSPECT run to create a fluence file.

The following options are possible:

MFLUX=0      special user-defined spectrum, look at the program.

MFLUX=1      Watt spectrum

=2      NBS-Uranium fission spectrum

=3      NBS-Californium fission spectrum.

=4      NBS URANIUM FISSION + 1/E PART, MATCHED AT 0.8 MEV

=5      the same as MFLUX=4 but thermal spectrum (MAXWELLIAN)

used  below 0.1ev

For more details look at the program.

type:     COMPLINK MSPECT     for compiling and linking

type:     MSPECT < MSPECT.INP to run the program with the input

file MSPECT.INP, look at it.

now you have the fluence file MSPECT.OUT available (user-defined)

type:     PLOT                to check the results.

************************** MRESBI ********************************

It converts the RESPDAT file (original response functions) to a library of

SAND-II type. The format of this library is the same as in the CS640

file of the REAL88 exercise /4/.

The program MRESBI creates a formatted and an unformatted response matrix

library. It converts a given pointwise response matrix with arbitrary

energy structure to either the SAND-II energy structure (640 groups),

or to the group structure calculated from the energies of the original

response matrix, or to the group structure given by an input group fluence

file. A log-log interpolation is performed (programs ECSI and GRATE from

the SUPERTOG program package).

input files needed :

MRESBI.INP        Look at this file

RESPDAT           The original response functions by A. Alevra

MSPECT.OUT        A 640 group fluence file is required, the

program calculates the integral cross sections

for that spectrum and writes the results to the

printer file

type:     COMPLINK MRESBI for compiling and linking

type:     MRESBI <  MRESBI.INP

to run the program with the input

file MRESBI.INP

You now have the following files available:

RESP48.FMT        Formatted response funtions in SAND-II format,

RESP48.BIN        Unformatted response functions in SAND-II format

MRESBI.PR1        Printer output file

MRESBI.48         OUTPUT fluence file (transformed MSPECT.OUT)

type      PLOT    to check the results.

In RESP48.FMT several additional 'cross sections' termed dosimetry reactions

are created e.g. :

PHI-THER

PHI-INT*

PHI-0.1*

PHI-1.0*

PHI-TOT*

Look at the printer output file MRESBI.PR1

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

You are now able to perform the adjustment (unfolding) runs.

You have available:

Activity file                 :        ACT  (possibly containing your own data)

Coarse binary cross

section file                  :        RESP48.BIN from MRESBI

You may plot spectrum files (MSPECT.OUT) or response functions from

the file RESP48.BIN by typing:   PLOF

and following the instructions given.

**************************** MSANDB ******************************

Modified SAND-II program.

The uncertainties of the reaction rates are taken into account.

input files:

MSANDB.INP        containing all the information

ACT               activity file

RESP48.BIN        binary cross section file

type:        COMPLINK MSANB   for compiling and linking

type:        MSANDB <  MSANDB.INP

to run the program with the

input file MSANDB.INP

now you have available:

MSANDB.PR1        Printer output

FLUX48.SAN        output fluence file

type:             PLOT        to check the results.

Note: You may also use the 640 group binary library for SAND-II

unfolding by changing the input in the program MRESBI.

************************* MSITER *********************************

The algorithm of this program is the same as realized in the STAY'SL code /10/

of Francis Perey, but some additional options are included. The most

important change is the fitting of a normalization factor of the spectrum

in the framework of a non-linear least-squares procedure.

There is a switch variable nstay to tell the program which mode the user

prefers:

nstay=0 : non-linear iteration, normalization constant is

calculated in each iteration,

=1 : staysl run with normalization constant c=1,

=2 : staysl run, c is calculated once due to STAY'SL

input files:

MSITER.INP        Input file containing all information.

In this example nstay=0 is used. You may compare

with STAY'SL runs of the original STAY'SL

program by setting nstay=2 in MSITER.INP.

ACT               Reaction rate file

RESP48.BIN        Unformatted coarse group response functions

FLUX48.SAN        Estimated pre-information fluence file.

type:     COMPLINK MSITER     for compiling and linking

type:     MSITER < MSITER.INP

to run the program

after the run you have the following files available:

MSITER.PR1        Printer output file,

FLUX48.MS         Fluence output file,

FLUXC48.MS        Binary output covariance file.

The covariance matrix of the output fluence can

also be found in the printer output file.

type:     PLOT                to check the results.

**************************** MIEKEB ******************************

Monte Carlo unfolding when no pre-information on the fluence is available.

Warning: In the few-channel unfolding you cannot pull rabbits out of a hat.

The output fluence of MIEKEB must therefore be considered with some caution.

In most cases the relative uncertainty of the fluence is more than 100%.

However, the output integral values and their correlations provide a

lot of useful information.

The organization of the program is similar to MSITER. You do 2 runs:

The first one is only to check the temperature parameter. The second

(long-time) run takes many hours.

First run:

input files:

MIEKEB.INP        Input file containing all information

ACT               Reaction rate file

RESP48.BIN        Unformatted response function file

type:     COMPLINK MIEKEB     for compiling and linking

type:     MIEKEB < MIEKEB.INP

to run the program

after the run you have the following files available:

MIEKEB.PR1        Printer output file,

FLUX48.MI1        Fluence output file,

FLUXC48.MI1       Binary output covariance file.

The covariance matrix of the output fluence can

also be found in the printer output file.

type:        PLOT             to check the results.

Second run:

type:     MIEKEB <  MIEKEB.IN2

to run the program with the second

input file.

comment: The value of the change parameter for MC-changes should be

chosen

that 50% of the changes are accpepted. The temperature parameter should

be chosen that CHI**2 is equal to the degrees of freedom. In the output a

value lambda is printed for the estimation of a new temperature parameter.

after the second run: type:  PLOT

*****************************************************************

7. FORMATS OF REACTION RATE FILES AND LIBRARIES

*****************************************************************

7.1 Spectrum Files

------------------

Each energy interval i is characterized by the left and right boundaries

Ei and Ei+1. The fluence in group i is represented by

INTEGRAL(F(E)dE) = fi (integrated over interval Ei, Ei+1)

The spectrum files are organized as follows:

line No. 1:  Comment line (less than 80 characters)

line No. 2:  MODE, MEV (two integer variables separated by Komma)

MODE = 0: Spectral point values of the fluence follow

= 1: mean spectral fluence values follow, i.e.

fi/(Ei+1-Ei)

= 2: group fluence values fi follow.

= 3: mean group fluence values per unit lethargy

follow, i.e. Fi/log (Ei+1/Ei)

MEV  = 0: The following energies are given in eV

= 1: The following energies are given in MeV

(default)

= 2: The following energies are given in keV.

line No. 3:  IL, IH, KG, E(KG+1), IUN        with:

IL:  first index e.g. 1

IH:  last index e.g. KG

KG:  number of groups (or points for MODE=0)

E(KG+1) last right energy boundary or dummy variable

for Mode=0

IUN=1: Uncertainties of the flux values follow.

This number is not used in the present version of the

programs

line No. 4 to line No. KG+3:

E(I), FLUX(I), SIG(I) with:

E(I): left boundary in the energy interval:

FLUX(I):Fluence value depending on MODE

SIG(I): Standard Deviation of FlUX(I), if IUN=1

7.2 Activity or Reaction Rate Files

-----------------------------------

Reaction rates correspond to flux densities.

Reaction events or reaction numbers correspond to fluences.

In reactor metrology, a reaction is characterized by two integer numbers

according to ENDF, the MAT(Material) and MT(specific reaction) numbers.

In the libraries used in the REAL intercomparisons /4/ a certain name is given

to such a MAT-MT combination. For instance, for the reaction 58Ni(n,p) there is

MAT=6433 (ENDF- Dosimetry file) and MT=103, the reaction is characterized by the

name NI58P in the cross section library IRDF.FMT or IRDF.BIN produced by the

MCROBI program.

For BONNER spheres, the names of the spheres are defined in the file RESPDAT,

the original response matrix file and are transformed to the binary library

by the MRESBI program.

The activity file is organized as follows:

line No. 1:   Comment line, less than 80 characters

line No. 2:   KA, ISO (Separated by komma)

KA: Number of detectors

ISO: ISO is only used by the program MFITIB for

shielding calculation

ISO=0 no shielding

ISO=1 shielding of the detectors, beam incidence

=2 shielding of the detectors, isotropic

incidence of the fluence

line No. 3 to line (KA+2):

NAM1, NAM2, ACT1, SIG1, MAT,MT, NAMCOV, THICK, THICKF

FORMAT(2A4,E10.3, F10.3, 1X, I6, 1X, I4, 1X, A2, E10.3, 1X, E10.3)

NAM1, NAM2: CHARACTER*4 values for the names of

the reactions, the cross sections

for these names should be found

in the library

ACT1, SIG1: Reaction rate and relative standard

deviation in %

The next two columns are only used in reactor metrology programs e.g. in the

UNC33M program

MAT, MT:    MAT, MT numbers for the reaction

NAMCOV:     CHARACTER*4 value for the shielding

material in the library. (Only

needed if ISO not equal to zero)

THICK:      Thickness of the cover per barn.

THICKF:     Thickness of the detector per barn

(for selfshielding)

Next line:    KD

KD:         Number of integral parameters to be

calculated by the unfolding code.

Next line and the following KD lines:

NAM1, NAM2: Names of integral quantities,

these names should be part of the

cross section library.

(See also programs MRESBI and MCROBI)

Last line:        NAM1, NAM2: Name of an integral parameter for direct

calculation of that parameter by the

program MSITER.

7.3 Library-Files

-----------------

Besides the original response matrix file for Bonner spheres and the

original ENDF or IRDF cross section files for reactor dosimetry reactions,

a library (formatted and unformatted) in the SAND-II structure (DOSCROSS

structure /15/) is used.

The structure of a library is as follows:

Line No. 1:  MPL  FORMAT(I10)

MPL: Number of energy boundaries, (one more than

energy groups)

Next lines:  MPL energy boundaries with FORMAT(1P8E10.3)

Next line:   NC

NC:  Number of cover materials in the library.

Next lines:  NC sets consisting of a line with the name of the

cover material (FORMAT(A2)), followed by the lines

for MPL-1 cover cross sections (FORMAT(1P8E10.3))

Next line:   NL

NL:  Number of cross section sets in the library

Next lines:  NL Blocks consisting of:

a line with S,F (FORMAT(2A4,2X,4A4))

S: short name of the reaction

F: 16 letter name of the reaction

A line with  CLAM, CN1, CN2, MAT, MT (FORMAT(1PE10.3,2A4,2X,2I10)

CLAM: Decay constant of the activated isotope (not

used in these programs)

CN1,CN2: Name of the isotope

MAT,MT:  ENDF numbers.

note: this line is only used by the reactor dosimetry

code UNC33M, for BONNER spheres these line contains

dummy variables.

the lines with MPL-1 cross sections (FORMAT(1P8E10.3)) follow.

The binary cross section file created from the formatted library has the same

structure.

8. MATHEMATICAL APPROACHES FOR MSANDB, MSITER AND MIEKE

-------------------------------------------------------------------------

A short review is given here of the mathematical foundation of the three

unfolding programs. For more detailed information the reader is referred to

the original literature. An overview is given in /16/.

8.1 SAND-II

In the MSANDB version of the SAND-II code /13/, the standard deviation of the

measured reaction rates is taken into account. In principle, it is assumed

that a number of reaction rates or events with standard deviations and

diagonal covariance matrix are measured.

Reaction rates A(I) (I=1,...,KA):

A(I)=SUM(1:M)   S(I,m)*Fm     I=1,...,KA                 (1)

where SUM(1:M) means summation, S(I,m) is the cross section of reaction No.I

in group m, Fm is the fluence rate in group m.

Logarithmic values are considered, leading to a chi**2:

CHI**2= SUM(1:KA) (LN(A0(I)-LN(A1(I))**2 /RHO(I)**2       (2)

where RHO(I) is the relative standard deviation of the measured A0(I), and

A1(I) are the calculated reaction rates obtained by means of Fm.

In the SAND-II formalism, the minimum of eq.(2) is reached in an

iteration procedure, where the iteration k and k+1 for Fm are given by

ln(Fm(k+1))=ln(Fm(k)) + SUM(1:KA)*

( ln(A0(I)/A1(I))*W(I,m)/RHO(I)**2

/SUM(1:KA) W(I,m)/RHO(I)**2

Where the following values are taken at the k-th iteration:

W(I,m) = S(I,m)*Fm(k)/A1(I)

A1(I)  = SUM(1:M) S(I,m1)*Fm1(k)

The result of the SAND-II iteration procedure depends on the first values

used for Fm(1).

8.2 MSITER

-------------

The description of the algorithm is given in chapter 1. Additional remarks

can be found in the program comments (see also /16/: The DIFMAZ program)

8.3 MIEKEB

-------------

The MIEKEB program can be used if pre-information on the fluence is not

available /16/.  In the formalism applied for the MIEKEB code the principle of

maximum entropy is used for the construction of a probability density P(F)

for a fluence vector F:

P(F) = const exp(-beta*CHI**2)

with

CHI**2 = (A0-S0*F)T * KA0(-1) * (A0-S0*F)

(A0: measured reaction rates, KA0: covariance matrix of A0, S0: cross

section matrix, F: fluence rate vector to be evaluated, KA0(-1): inverse)

beta is a parameter to be determined from the condition that the expectation

value of CHI**2 should be equal to the number of degrees of freedom

involved,

which is equal to the number of detectors investigated. By means of the

code, expectation values of F (termed <F>) and the covariance matrix

<FF>-<F><F> are calculated.

The method applied is that of importance sampling. In the MIEKE code

uncertainties of the cross sections cannot be taken into account.

However, in the sense of an iteration procedure, after the first run a

new "covariance matrix of activities" can be evaluated.

New  =  old +   TT

KA0  = KA0  + F1*KS*F1

where F1 are the fluence results of the first run and KS is the covariance

matrix of cross sections. Proceeding with possible further runs, the

uncertainty propagation of the cross section can be performed.

It should be mentioned that the MIEKEB code is rather time-consuming.

It allows up to 100 energy groups for the fluence, but in worst cases needs

more than 30 hours PC time with an 80387 coprozessor at 20 MHz.

*****************************************************************************

*************************************

9. COMMENTS ON THE PROGRAMS

*************************************

LOOK AT THE SOURCE CODES

***************************************

10. REFERENCES

***************************************

/1/   E. M. Zsolnay, H. J. Nolthenius, L. R. Greenwood:

Reference data file for neutron spectrum adjustment and

related radiation damage calculations.

Proc. of the 7th ASTM-EURATOM Symposium on Reactor Dosimetry,

Strasbourg 1990, Ed. by G. Tsotridis, R. Dierckx and P. D'Hondt. Kluwer

Academic publishers, Dordrecht, Netherlands 1992

/2/   W. L. Zijp, E. M. Zsolnay, H. J. Nolthenius, E. J. Szondi:

Final report on the REAL-84 exercise.

Report ECN-212, ECN Petten 1988.

/3/   H. J. Nolthenius: Information for the REAL-88 exercise.

Report ECN 88-023, ECN Petten 1988

/4/   H. J. Nolthenius: Results of the REAL-88 exercise.

Report ECN 89-095, ECN Petten 1989

/5/   H. Ch. Rieffe: FITOCO - A program for the conversion of fine group

flux density and cross-section data to coarse group values

Report ECN-92, ECN Petten 1981

/6/   H. J. Nolthenius: Computer programs for calculating group cross-

section covariances from uncertainty parameters in the ENDF-B/V

Format, UNC32/33

Report ECN-202, ECN Petten 1987

/7/   D. E. Cullen, P. K. McLaughlin:

The international reactor dosimetry file (IRDF-85)

Report IAEA-NDS-41, Rev. 1, IAEA Vienna 1985.

/8/   N. P. Kocherov, P. K. McLaughlin:

The international reactor dosimetry file (IRDF-90)

Report IAEA-NDS-141, Rev. 0, IAEA Vienna 1990.

/9/   A. V. Alevra: Private Communication, PTB Braunschweig 1990

/10/  F. G. Perey: Least-squares dosimetry unfolding: The program STAY'SL

Report ORNL/TM-6062, ENDF-254, Oak Ridge National Laboratory 1977

/11/  A detailed description of the algorithm of MSITER may be found in ref. /16/

in the chapter concerning least-squares programs.

/12/  J. J. Wagschal, Y. Yeivin: Uses and abuses of adjustment-

a critical review of common practice.(see ref. /1/)

/13/  W. N. McElroy et al.: A computer automated iteration method for

neutron flux spectra determination

Report AFWL-TR-67-41, Vol. I...IV, (air force weapons laboratory,

New Mexico 1967

/14/  M. Matzke: Unfolding and radiation damage prediction with and without

a priori information. Proc. 6th ASTM-EURATOM symposium on reactor

dosimetry 1987. ASTM-STP 1001, Philadelphia 1989

/15/  W. L. Zijp, H. J. Nolthenius, H. Ch. Rieffe:

Cross-section library DOSCROSS81

Report ECN-111, ECN Petten 1981

/16/  M. Matzke: Unfolding of Pulse Height spectra: The HEPRO program system.

Report PTB-N-19, Physikalisch-Technische Bundesanstalt, Braunschweig,

1994, ISBN 3-89429-543-0

/17/  E. J. Szondi: The Group version of the International Reactor Dosimetry File

IRDF-90 for Use in the Neutron Metrology File NMF-90 (IRDF-90/NMF-G).

Institute of Nuclear Techniques, Technical University Budapest, Report

INDC(HUN)-31, 1994.

/18/  E. J. Szondi, H. J. Nolthenius: Additional data and information

for the REAL-84 exercises. Report INDC(NDS)-167. IAEA, Vienna, 1985

- 36 -